SUPREMA ACCESS CONTROL DEVICE - FaceStation F2

TECHNICAL SPECIFICATIONS

2022-01-10

PART 1 - GENERAL

The intent of this document is to specify the minimum criteria for the design, supply, installation, and commissioning of the FaceStation F2.

SUMMARY

Section includes a biometric reader and door controller with Ethernet network connectivity.

Product - An IP enabled biometric reader and door controller, capable of scanning and registering faces, fingerprints (optional), RFID cards, and mobile access cards, managing users and controlling access.

REFERENCE

- Standards

- IEEE 802.3/802.3u Ethernet Standards

- FCC - Code of Federal Regulations, Part 15, Class B

- Conformity for Europe (CE) - R&TTE Directive 1999/5/EC

- Korea Certification (KC)

- Registration, Evaluation, Authorization and Restriction of Chemicals (REACH) - (EC No. 1907/2006)

- The Waste Electrical and Electronic Equipment (WEEE) - Directive 2012/19/EU

- ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

- International Electrotechnical Commission (IEC) – Ingress Protection Rating IP65

  - SUBMITTALS

QUALIFICATIONS

- Manufacturer shall be ISO 9001 certified with a minimum of five years’ experience in producing access control equipment.

Installers shall be trained by the Manufacturer to install, configure and commission the access control system.

WARRANTY

- Manufacturer shall provide a limited (  ) month warranty for the product to be free of defect in material and workmanship.

END OF SECTION

PART 2 - PRODUCTS

EQUIPMENT

- Manufacturer 
Suprema Inc. 
17F Parkview Office Tower, Jeongja, Bundang, Seongnam, Gyeonggi, 463-863, Republic of Korea
Tel: 82-31-783-4502, Fax: 82-31-783-4503, www.supremainc.com
support@supremainc.com

Model(s) name: FaceStation F2

- Part Number: FSF2

- Sub Model

- FSF2-ODB: Face recognition & Optical fingerprint, EM & MIFARE dual frequency, Mobile Card (NFC, BLE) supported

- FSF2-DB: Face recognition, EM & MIFARE dual frequency, Mobile Card (NFC, BLE) supported

- FSF2-AB: Face recognition, EM, MIFARE, iCLASS, FeliCa multi frequency, Mobile Card (NFC, BLE) supported

Alternates: NONE

DESCRIPTION

- The biometric reader and door controller (“reader/controller”) shall be an IP-enabled device capable of scanning faces, fingerprints (optional), RFID cards, and mobile access cards, managing users and controlling access.

FEATURES

- Time Attendance and Access Control device based on Android 6.1 Oreo

- Dual Core 1.8 GHz and Quad Core 1.4 GHz with 2GB RAM

- Mobile Access card support (NFC, BLE)

- Multi-class RFID card reading

- Anti-Spoofing technology

- Photo enrollment support

- Upload photo or drag & drop

- Enrollment through email link

- Bulk enrollment through CSV import

- Integration with DB/ERP/HEMS for photo importing

- Recognition with masked face

- IP65, Dust & Waterproof

- 7” IPS color LCD with capacitive touch screen

- TCP/IP, RS-485, Wiegand, TTL, Relay, USB, Tamper

- Intuitive Graphical User Interface (GUI) system

- FBI PIV and FBI Mobile ID FAP20 certified (FSF2-ODB only)

SPECIFICATIONS

| Category | Feature | Specification |
| --- | --- | --- |
| Credential | Biometric | - FSF2-DB, FSF2-AB: Face, - FSF2-ODB: Face, Fingerprint |
|  | RF Option | - FSF2-DB, FSF2-ODB: 125kHz EM & 13.56MHz MIFARE, MIFARE Plus, DESFire, DESFire EV1/EV2/EV3*, FeliCa - FSF2-AB: 125kHz EM & 13.56MHz MIFARE, MIFARE Plus, DESFire, DESFire EV1/EV2/EV3*, FeliCa, iCLASS SE/SR/Seos |
|  | RF read range | EM/MIFARE/DESFire: 50 mm, FeliCa: 30 mm * RF read range will vary depending on the installation environment. |
|  | Mobile | Frequency: 13.56MHz NFC & 2.4GHz BLE |
|  |  | Certificates: ISO 27001 |
|  |  | Encryption: AES-256 |
|  |  | Compatible devices: iOS 7.0 above, Android 9.0 above |
| General | CPU | 1.8 GHz Dual Core + 1.4 GHz Quad Core |
|  | Memory | 16GB Flash + 2GB RAM |
|  | Crypto chip | Supported |
|  | LCD type | 7” IPS color LCD |
|  | LCD resolution | 800 x 1280 pixels |
|  | Sound | 16bit |
|  | Operating temperature | -20°C to50°C |
|  | Storage temperature | -40°C to 70°C |
|  | Operating humidity | 0% to 80%, non-condensing |
|  | Storage humidity | 0% to 90%, non-condensing |
|  | Camera | 2MP 2EA |
|  | Dimension (W x H x D) | - FSF2-DB, FSF2-AB: 119.8 x 233 x 23.5 (mm) - FSF2-ODB: 119.8 x 268.3 x 49.6 (mm) |
|  | Weight | Device - FSF2-ODB: 670g - FSF2-DB, FSF2-AB: 585g |
|  |  | Bracket (Including washer and bolt) - FSF2-ODB: 205g - FSF2-DB, FSF2-AB: 181g |
|  | Certificates | CE, FCC, KC, RoHS, REACH, WEEE |
| Face | Recognition Distance | 0.5 ~ 1.3 m |
|  | Recognition Height | 1.4 ~ 1.9 m |
|  | Accuracy rate | ≥ 99% |
|  | Matching speed | Less than 0.5 sec |
| Fingerprint (FSF2-ODB) | Image dimension | 300 x 400 pixels |
|  | Resolution | 500 dpi |
|  | Template | SUPREMA / ISO 19794-2 / ANSI 378 |
|  | Extractor / Matcher | MINEX certified and compliant |
|  | Sensor Certificates | FBI PIV and FBI Mobile ID FAP20 |
|  | Live Fingerprint Detection | Supported (SW-based) |
| Capacity | Max. User | 100,000** |
|  | Max. Credential (1:N) | - Face: 50,000 - Fingerprint (FSF2-ODB): 100,000 |
|  | Max. Credential (1:1) | - Face: 100,000 - Fingerprint (FSF2-ODB): 100,000 - Card: 100,000 - PIN: 100,000 |
|  | Max. Text Log | 5,000,000 |
|  | Max. Image Log | 50,000 |
| Interface | Ethernet | 10/100 Mbps, auto MDI/MDIX |
|  | RS-485 | 1ch Host or Slave (Selectable) |
|  | Wiegand | 1ch Input, 1ch Output |
|  | TTL input | 2ch Inputs |
|  | Relay | 1 Relay |
|  | USB | USB 2.0 (Host) |
|  | Tamper | Supported |
| Electrical | Power | - Power: DC 12 V–DC 24 V (Max. 2.5 A) - Adapter recommended specifications: DC 24 V or DC 12 V (± 10%) with a minimum of 2,500 mA*** * Use a 24V/2.5A adapter. Must follow the product manual when using a 12V adapter. |
|  | Switch input VIH | Min. 3V Max. 5V |
|  | Switch input VIL | Max. 1V |
|  | Switch Pull-up resistance | 4.7kΩ (The input pots are pulled up with 4.7kΩ.) |
|  | Wiegand output VOH | More than 4.8V |
|  | Wiegand output VOL | Less than 0.2 V |
|  | Wiegand output Pull-up resistance | Internally pulled up with 1 kΩ |
|  | Relay | 2 A @ 30 VDC Resistive load 1 A @ 30 VDC Inductive load |
| Platform | BioStar 2 | Supported |

* These documents must be kept completely confidential.

* DESFire EV2/EV3 cards are supported by having backward compatibility of DESFire EV1 cards. CSN and smart card functions are compatible with FaceStation F2.

** The number of users registered without having any credential data

*** Adapter is sold separately. Refer to the recommended specifications.

END OF SECTION

PART 3 - EXECUTION

INSTALLER

- Contractor personnel shall comply with all applicable state and local licensing requirements.

PREPARATION

- Contractor shall avoid locating the reader/controller in a location subject to direct sunlight, dust or soot.

IP addressing shall be coordinated with the Owner’s responsible IT personnel.

STORAGE

- The device shall be stored in an environment where temperature is in the range of -40°C to 70°C.

- The device shall be stored in an environment where humidity is in the range of 0% to 90%, non-condensing.

INSTALLATION

- The device shall be installed in an environment where temperature is in the range of -20°C to 50°C.

- The device shall be installed in an environment where humidity is in the range of 0% to 80%, non-condensing.

- All wires shall be run through conduit to prevent failure caused by rodent damage.

Connections between card readers and a door controller shall not exceed 100 meters.

All peripheral devices shall be grounded.

Keep at least 10 cm distance between the devices when install multiple devices

To avoid RF interference, a minimum separation distance must be maintained.

| - Wall thickness | - Distance |
| --- | --- |
| - 100 mm | - 200 mm |
| - 120 mm | - 180 mm |
| - 150 mm | - 150 mm |

EXAMINATION

- All network connections to the reader/controller shall be tested for proper levels of performance.

END OF SECTION