SUPREMA SECURE MULTI-INPUT MODULE– IM-120

TECHNICAL SPECIFICATIONS

2021-12-02

PART 1 - GENERAL

The intent of this document is to specify the minimum criteria for the design, supply, installation, and commissioning of the IM-120.

SUMMARY

A multi-input module, capable of instant relay behavior for detected inputs in real-time.

REFERENCE

- Standards

- FCC—Code of Federal Regulations, Part 15, Class A

- Conformity for Europe (CE)—Equipment Directive (RED) 2014/53/EU

- Korea Certification (KC)

- Registration, Evaluation, Authorization, and Restriction of Chemicals (REACH)—EC No. 1907/2006

- The Waste Electrical and Electronic Equipment (WEEE)—Directive 2012/19/EU

  - SUBMITTALS

QUALIFICATIONS

- Manufacturer shall be ISO 9001 certified with a minimum of five years’ experience in producing access control equipment.

Installers shall be trained by the Manufacturer to install, configure and commission the access control system.

WARRANTY

- Manufacturer shall provide a limited (  ) month warranty for the product to be free of defects in material and workmanship.

END OF SECTION

PART 2 - PRODUCTS

EQUIPMENT

- Manufacturer 
Suprema Inc. 
17F Parkview Office Tower, Jeongja, Bundang, Seongnam, Gyeonggi, 463-863, Republic of Korea
Tel: 82-31-783-4502, Fax: 82-31-783-4503, www.supremainc.com

Model(s) name: Input Module

- Part Number: IM-120

Alternates: NONE

DESCRIPTION

- A multi-input module, capable of instant relay behavior for detected inputs in real-time by connecting to BioStar 2.

- Supports 12 supervised inputs per module.

FEATURES

- Linkage between inputs and relays where the relay operates immediately when an input signal is detected in real-time.

- Detects 4 states such as ON, OFF, Open, and Short with 12 supervised inputs.

- Supports offline operation.

- Provides various interfaces: Supervised Input, Relay, RS-485, AUX Input, Tamper.

SPECIFICATIONS

| Category | Feature | Specification |
| --- | --- | --- |
| General | CPU | Cortex M3 72 MHz |
|  | Memory | 512 KB Flash + 64 KB SRAM |
|  | LED | Multi-color - Power x 1 - Status x 1 - RS-485 x 2 - Input x 12 - Relay x 2 - AUX x 2 - Tamper x 1 |
|  | Operating temperature | -20°C–60°C |
|  | Storage temperature | -40°C–70°C |
|  | Operating humidity | 0 %–95 %, non-condensing |
|  | Storage humidity | 0 %–95 %, non-condensing |
|  | Dimension (W x H x D) | 90 mm x 190 mm x 21 mm |
|  | Weight | 203 g |
|  | Certificates | CE, FCC, KC, RoHS, REACH, WEEE |
| Capacity | Max. Text Log | 10ea per port* |
| Interface | Supervised Input | 12 ch (TTL Input selectable) |
|  | RS-485 | 1 ch |
|  | Relay | 2 relays |
|  | AUX Input | 3 ch (Power, Battery, Tamper) |
| Electrical | Power | - Power: DC 12 V (Max. 130 mA) or DC 24 V (Max. 82 mA) - Adapter recommended specifications: DC 12 V (± 10%) with a minimum of 1 A or DC 24 V (±10%) with a minimum of 1 A ** |
|  | Switch input VIH | Max. 5 V (Dry Contact) |
|  | Relay | 5 A @ 30 VDC Resistive load |
| Platform | BioStar 2 | Supported |

* These documents must be kept completely confidential.

* Product specifications are subject to change without notice to improve.

* Logs up to 10 events that occurred during network disconnection. If more than 10 events occur, they will be deleted in the order of event logs that occurred first.

** Adapter is sold separately. Refer to the recommended specifications.

END OF SECTION

PART 3 - EXECUTION

INSTALLER

- Contractor personnel shall comply with all applicable state and local licensing requirements.

PREPARATION

- Contractor shall avoid locating the reader/controller in a location subject to direct sunlight, dust, or soot.

STORAGE

- The device shall be stored in an environment where temperature is in the range of -40°C–70°C.

- The device shall be stored in an environment where humidity is in the range of 0 % – 95 %, non-condensing.

INSTALLATION

- The device shall be installed in an environment where temperature is in the range of -20°C – 60°C.

- The device shall be installed in an environment where humidity is in the range of 0 % – 95 %, non-condensing.

- All wires shall be run through conduits to prevent failure caused by rodent damage.

Connections between card readers and a door controller shall not exceed 100 meters.

All peripheral devices shall be grounded.

EXAMINATION

- All network connections to the reader/controller shall be tested for proper levels of performance.

END OF SECTION