SUPREMA SECURE MULTI-OUTPUT MODULE - OM-120

TECHNICAL SPECIFICATIONS

2018-07-10

PART 1 - GENERAL

The intent of this document is to specify the minimum criteria for the design, supply, installation, and commissioning of the OM-120.

SUMMARY

A multi-output module, capable of connected to Biometric/RFID devices to control access to zones or floors.

REFERENCE

- Standards

FCC - Code of Federal Regulations, Title 47, Part 15, Class B

  - SUBMITTALS

QUALIFICATIONS

- Manufacturer shall be ISO 9001 certified with a minimum of five years’ experience in producing access control equipment.

Installers shall be trained by the Manufacturer to install, configure and commission the access control system.

WARRANTY

- Manufacturer shall provide a limited (  ) month warranty for the product to be free of defect in material and workmanship.

END OF SECTION

PART 2 - PRODUCTS

EQUIPMENT

- Manufacturer 
Suprema Inc. 
17F Parkview Office Tower, Jeongja, Bundang, Seongnam, Gyeonggi, 463-863, Republic of Korea
Tel: 82-31-783-4502, Fax: 82-31-783-4503, www.supremainc.com
support@supremainc.com

Model(s): OM-120

Alternates: NONE

DESCRIPTION

- The multi-output module, capable of configuring and controlling zones or floors with BioStar 2.

- Support 12 relay outputs per module.

FEATURES

- Control up to 192 floors per elevator with BioStar 2 when connected as daisy chains via RS-485.

- Capable of setting the floor access privilege in BioStar 2 when connected Biometric/RFID devices.

- Switchable on-board RS-485 termination.

- Interlock with Anti-passback zones and Fire Alarm zones, manage to occur specific outputs when an alarm is triggered.

- RS-485, AUX Input, Relay.

SPECIFICATIONS

| Category | Feature | Specification |
| --- | --- | --- |
| General | CPU | Cortex M3 72MHz |
|  | Memory | Flash 128KB, SRAM 20KB |
|  | LED | Multi-color - Power 1 ea - Relays 12 ea - RS-485 2 ea - AUX IN 2 ea - Status 1 ea |
|  | Operating temperature | -20°C ~ 60°C |
|  | Storage temperature | -40°C ~ 70°C |
|  | Operating humidity | 0% ~ 95%, non-condensing |
|  | Storage humidity | 0% ~ 95%, non-condensing |
|  | Dimension (W x H x D) | 90 mm x 190 mm x 21 mm |
|  | Weight | 300 g |
|  | Certificates | CE, FCC, KC, RoHS, REACH, WEEE |
| Interface | RS-485 | 1ch |
|  | AUX input | 2ch Dry Contact Input |
|  | Relay | 12 Relays |
| Electrical | Power | Voltage: 12 VDC Current: Max. 1 A |
|  | Switch input VIH | Max. 5 V (Dry Contact) |
|  | Relay | Voltage: Max. 30 VDC Current: Max. 5 A |

END OF SECTION

PART 3 - EXECUTION

INSTALLER

- Contractor personnel shall comply with all applicable state and local licensing requirements.

PREPARATION

- Contractor shall avoid locating the reader/controller in a location subject to direct sunlight, dust or soot.

STORAGE

- The device shall be stored in an environment where temperature is in the range of -40°C - +70°C.

- The device shall be stored in an environment where humidity is in the range of 0% - 95%, non-condensing.

INSTALLATION

- The device shall be installed in an environment where temperature is in the range of -20°C - 60°C.

- The device shall be installed in an environment where humidity is in the range of 0% - 95%, non-condensing.

- All wires shall be run through conduit to prevent failure caused by rodent damage.

Connections between card readers and a door controller shall not exceed 100 meters.

All peripheral devices shall be grounded.

EXAMINATION

- All connections to the reader/controller shall be tested for proper levels of performance.

END OF SECTION