SUPREMA ACCESS CONTROL DEVICE - Secure I/O 2

TECHNICAL SPECIFICATIONS

2016-05-31

PART 1 - GENERAL

The intent of this document is to specify the minimum criteria for the design, supply, installation, and commissioning of the Secure I/O 2.

SUMMARY

Section includes a RF reader and door controller with RS-485 connectivity.

Product - An I/O expanding device, capable of connecting with biometric/RF reader via RS-485 and controlling relays and external signal input.

REFERENCE

- Standards

FCC - Code of Federal Regulations, Title 47, Part 15, Class B

  - SUBMITTALS

QUALIFICATIONS

- Manufacturer shall be ISO 9001 certified with a minimum of five years’ experience in producing access control equipment.

Installers shall be trained by the Manufacturer to install, configure and commission the access control system.

WARRANTY

- Manufacturer shall provide a limited (  ) month warranty for the product to be free of defect in material and workmanship.

END OF SECTION

PART 2 - PRODUCTS

EQUIPMENT

- Manufacturer 
Suprema Inc. 
17F Parkview Office Tower, Jeongja, Bundang, Seongnam, Gyeonggi, 463-863, Republic of Korea
Tel: 82-31-783-4502, Fax: 82-31-783-4503, www.supremainc.com
support@supremainc.com

Model(s): Secure I/O 2

Alternates: NONE

DESCRIPTION

- The I/O device shall be an I/O expanding device capable of connecting with biometric/RF reader via RS-485 and controlling relays and external signal input.

FEATURES

- Encrypted communication with connected device

- Compact size

- Easy to installation

Supports 1 relay, 2 external inputs

- RS-485, TTL, Relay interface

SPECIFICATIONS

| General | CPU | Cortex M 32MHz |
| --- | --- | --- |
|  | Memory | 128KB Flash + 20KB RAM |
|  | LED | Multi-Color |
|  | Operating temperature | -20°C ~ 50°C |
|  | Storage temperature | -40°C ~ 70°C |
|  | Operating humidity | 0% ~ 80%, non-condensing |
|  | Storage humidity | 0% ~ 90%, non-condensing |
|  | Dimension (W x H x D) | 36 mm x 65 mm x 18 mm |
|  | Weight | Device: 37 g |
|  |  | Bracket: 1 g |
|  | Certificates | CE, FCC, KC, RoHS |
| Interface | RS-485 | 1ch Slave |
|  | TTL Input | 2 Inputs |
|  | Relay | 1 Relay |
| Electrical | Power | Voltage: DC 12 V Current: Max. 200 mA |
|  | Switch input VIH | Min. 3V Max. 5V |
|  | Switch input VIL | Max. 1V |
|  | Switch Pull-up resistance | 4.7kΩ (The input ports are pulled up with 4.7kΩ.) |
|  | Relay | Voltage: Max. 30 VDC Current: Max. 2 A |

END OF SECTION

PART 3 - EXECUTION

INSTALLER

- Contractor personnel shall comply with all applicable state and local licensing requirements.

PREPARATION

- Contractor shall avoid locating the reader/controller in a location subject to direct sunlight, dust or soot.

STORAGE

- The device shall be stored in an environment where temperature is in the range of -40°C - +70°C.

- The device shall be stored in an environment where humidity is in the range of 0% - 90%, non-condensing.

INSTALLATION

- The device shall be installed in an environment where temperature is in the range of -20°C - 50°C.

- The device shall be installed in an environment where humidity is in the range of 0% - 80%, non-condensing.

- All wires shall be run through conduit to prevent failure caused by rodent damage.

Connections between card readers and a door controller shall not exceed 100 meters.

All peripheral devices shall be grounded.

EXAMINATION

- All network connections to the reader/controller shall be tested for proper levels of performance.

END OF SECTION