SUPREMA TIME AND ATTENDANCE / WORKFORCE MANAGEMENT

MULTIMEDIA WORKSTATION - OMNIS

TECHNICAL SPECIFICATIONS

2019-11-05

PART 1 - GENERAL

The intent of this document is to specify the minimum criteria for the design, supply, installation, and commissioning of the OMNIS.

SUMMARY

Section includes a time attendance and workforce management terminal with the Android open platform.

Product - A biometric terminal, capable of integrating with the time attendance and workforce management applications.

REFERENCE

- Standards

- IEEE 802.3 Ethernet Standards

- FCC - Code of Federal Regulations, Title 47, Part 15, Class B

- Conformity for Europe (CE) – R&TTE Directive 1999/5/EC

- Korea Certification (KC)

- Registration, Evaluation, Authorization and Restriction of Chemicals (REACH) - (EC No. 1907/2006)

- The Waste Electrical and Electronic Equipment (WEEE) - Directive 2012/19/EU

  - SUBMITTALS

QUALIFICATIONS

- Manufacturer shall be ISO 9001 certified with a minimum of five years’ experience in producing access control equipment.

Installers shall be trained by the Manufacturer to install, configure and commission the access control system.

WARRANTY

- Manufacturer shall provide a limited (  ) month warranty for the product to be free of defect in material and workmanship.

END OF SECTION

PART 2 - PRODUCTS

EQUIPMENT

- Manufacturer 
Suprema Inc. 
17F Parkview Office Tower, Jeongja, Bundang, Seongnam, Gyeonggi, 463-863, Republic of Korea
Tel: 82-31-783-4502, Fax: 82-31-783-4503, www.supremainc.com
http://support.supremainc.com

Model(s): OMNIS

Alternates: NONE

DESCRIPTION

- The biometric terminal shall be an IP-enable device capable of scanning fingerprints and RFID cards for managing the time attendance and workforce.

FEATURES

- Time Attendance and Workforce Management terminal based on Android 8.0 Oreo

- Open platform SVP Android SDK for easy integration of a customized user interface

- Quad Core 1.0 GHz with 1GB RAM

- Live Finger Detection (LFD)

- Multi-class RFID card reading

- Built-in Wi-Fi module (IEEE 802.11 b/g)

- NIST MINEX certified and compliant

- 5” TFT color LCD with capacitive touch screen

- CMOS 2M pixels camera

- Wi-Fi, Wiegand Input, TTL, Relay, USB, SD Card, PoE, Tamper

SPECIFICATIONS

| Category | Feature | Specification |
| --- | --- | --- |
| Credential | Biometric | Fingerprint |
|  | RF Option | OMNIS-E: 125kHz EM |
|  |  | OMNIS-H: 125kHz HID Prox |
|  |  | OMNIS-I: 13.56MHz iCLASS SE/SR/Seos, NFC |
|  |  | OMNIS-M: 13.56MHz MIFARE, MIFARE Plus, DESFire/EV1, FeliCa, NFC |
|  | RF read range* | MIFARE: 50 mm, DESFire: 50 mm, FeliCa: 30 mm * RF read range will vary depending on the installation environment. |
| General | CPU | 1.0 GHz Quad Core |
|  | Memory | 8GB Flash + 1GB RAM |
|  | LCD type | 5” color TFT LCD |
|  | LCD resolution | 480 x 854 pixels |
|  | LED | Multiple colors |
|  | Sound | 24 bit/Voice DSP (echo cancellation) |
|  | Operating temperature | -20°C ~ 50°C |
|  | Storage temperature | -40°C ~ 70°C |
|  | Operating humidity | 0% ~ 80%, non-condensing |
|  | Storage humidity | 0% ~ 90%, non-condensing |
|  | Camera type | CMOS 2M pixels |
|  | Camera resolution | 1600 x 1200 pixels |
|  | Camera angle | Diagonal 122°, Horizontal 64.7°, Vertical 103.3° |
|  | Dimension (W x H x D) | 155 mm x 155 mm x 40 mm |
|  | Weight | Device: 444g |
|  |  | Bracket: 89g (Including washer and bolt) |
|  | Certificates | CE, FCC, KC, RoHS, REACH, WEEE |
| Fingerprint | Image dimension | 272 x 320 pixels |
|  | Image bit depth | 8bit, 256 grayscale |
|  | Resolution | 500 dpi |
|  | Template | SUPREMA / ISO 19794-2 / ANSI 378 |
|  | Extractor / Matcher | MINEX certified and compliant |
|  | LFD | Supported |
| Capacity | Max. User (1:1) | 50,000 |
|  | Max. User (1:N) | 20,000 |
|  | Max. Template (1:1) | 100,000 * Two templates per finger |
|  | Max. Template (1:N) | 40,000 * Two templates per finger |
| Interface | Wi-Fi | Supported (Built-in, IEEE 802.11 b/g) |
|  | Ethernet | Supported (10/100 Mbps, auto MDI/MDI-X) |
|  | RS-485 | Not supported |
|  | RS-232 | Not supported |
|  | Wiegand | 1ch Input |
|  | TTL input | 2ch Inputs |
|  | Relay | 2 Relays |
|  | USB | USB 2.0 (Host) |
|  | SD Card | microSD card (Supports up to 32GB) |
|  | PoE | Supported (IEEE 802.3af compliant) |
|  | Tamper | Supported |
|  | Analog intercom | Not supported |
| Electrical | Power | Voltage: DC 12V Current: Max. 850 mA |
|  | Switch input VIH | Min. 3V Max. 5V |
|  | Switch input VIL | Max. 1V |
|  | Switch Pull-up resistance | 4.7kΩ (The input ports are pulled up with 4.7kΩ.) |
|  | Relay | Voltage: Max. 30VDC Current: Max. 1A |

END OF SECTION

PART 3 - EXECUTION

INSTALLER

- Contractor personnel shall comply with all applicable state and local licensing requirements.

PREPARATION

- Contractor shall avoid locating the device in a location subject to direct sunlight, dust or soot.

IP addressing shall be coordinated with the Owner’s responsible IT personnel.

STORAGE

- The device shall be stored in an environment where temperature is in the range of -40°C - +70°C.

- The device shall be stored in an environment where humidity is in the range of 0% - 90%, non-condensing.

INSTALLATION

- The device shall be installed in an environment where temperature is in the range of -20°C - 50°C.

- The device shall be installed in an environment where humidity is in the range of 0% - 80%, non-condensing.

- All wires shall be run through conduit to prevent failure caused by rodent damage.

All peripheral devices shall be grounded.

To avoid RF interference, a minimum separation distance must be maintained.

| - Wall thickness | - Distance |
| --- | --- |
| - 100 mm | - 500 mm |
| - 120 mm | - 400 mm |
| - 150 mm | - 300 mm |

EXAMINATION

- All network connections to the device shall be tested for proper levels of performance.

END OF SECTION