SUPREMA OUTDOOR COMPACT RFID READER – XPass D2

TECHNICAL SPECIFICATIONS

2024-02-20

PART 1 - GENERAL

The intent of this document is to specify the minimum criteria for the design, supply, installation, and commissioning of the XPass D2.

SUMMARY

An OSDP enabled RF reader, capable of scanning and registering RFID cards.

REFERENCE

- Standards

- Conformity for Europe (CE)—Equipment Directive (RED) 2014/53/EU

- UK Conformity Assessed (UKCA)

- Korea Certification (KC)

- FCC - Code of Federal Regulations, Title 47, Part 15, Class B

- Industry Canada (IC)

- Regulatory Compliance Mark (RCM)

- Bluetooth SIG

- Registration, Evaluation, Authorization and Restriction of Chemicals (REACH) - (EC No. 1907/2006)

- The Waste Electrical and Electronic Equipment (WEEE) - Directive 2012/19/EU

- ANSI / IEC60529 – Degrees of Protection Provided by Enclosures

- International Electrotechnical Commission (IEC) – Ingress Protection Rating IP65

- International Electrotechnical Commission (IEC) – Ingress Protection Rating IP67

- IEC 62262 - Degrees of protection provided by enclosures for electrical equipment against external mechanical impacts IK08.

  - SUBMITTALS

QUALIFICATIONS

- Manufacturer shall be ISO 9001 certified with a minimum of five years’ experience in producing access control equipment.

Installers shall be trained by the Manufacturer to install, configure, and commission the access control system.

WARRANTY

- Manufacturer shall provide a limited (  ) month warranty for the product to be free of defect in material and workmanship.

END OF SECTION

PART 2 - PRODUCTS

EQUIPMENT

- Manufacturer 
Suprema Inc. 
17F Parkview Office Tower, 248, Jeongjail-ro, Seongnam-si, Gyeonggi-do, 13554, Republic of Korea
Tel: 82-31-783-4502, Fax: 82-31-783-4503, https://www.supremainc.com
https://support.supremainc.com

Model(s): XPass D2

- Part Number: XPD2

Alternates: NONE

DESCRIPTION

- The RF reader which is an RS-485(OSDP) enabled device capable of scanning RFID cards and mobile cards.

- Provides a numeric keypad for inputting a PIN or Weigand card ID. (XPD2-GKDB only)

FEATURES

- Multi-RFID card reading

- Mobile Access card support (NFC, BLE)

- IP65, IP67, Dust & Waterproof

- IK08, Vandal proof

- Mullion type design (XPD2-MDB)

- Gangbox type design (XPD2-GDB)

- Gangbox type design with a numeric keypad (XPD2-GKDB)

- RS-485, Wiegand, Tamper

- OSDP V2 Compliant

SPECIFICATIONS

| Category | Feature | Specification |
| --- | --- | --- |
| Credential | RF Option | 125kHz EM & 13.56MHz MIFARE, MIFARE Plus, DESFire, DESFire EV1/EV2/EV31), FeliCa |
|  | RF Read Range2) | EM/MIFARE/DESFire: 50 mm, FeliCa: 30 mm |
|  | Mobile | NFC, BLE |
| General | CPU | 80 MHz |
|  | Memory | 512 KB Flash + 160 KB RAM |
|  | Crypto Chip | Supported |
|  | LED | Multi-color |
|  | Sound | Multi-tone Buzzer |
|  | Operating Temperature | -35 °C ~ 65 °C |
|  | Storage Temperature | -40 °C ~ 70 °C |
|  | Operating Humidity | 0 % ~ 95 %, non-condensing |
|  | Storage Humidity | 0 % ~ 95 %, non-condensing |
|  | Dimension (W x H x D) | - XPD2-MDB: 48 x 144.7 x 27 (mm) - XPD2-GDB: 80 x 130 x 25 (mm) - XPD2-GKDB: 80 x 130 x 25 (mm) |
|  | Weight | Device - XPD2-MDB: 117 g - XPD2-GDB: 140 g - XPD2-GKDB: 145 g |
|  |  | Bracket - XPD2-MDB: 30 g (Including washer and bolt) - XPD2-GDB: 52 g (Including washer and bolt) - XPD2-GKDB: 52 g (Including washer and bolt) |
|  | IP Rating | IP65, IP67 |
|  | IK Rating | IK08 |
|  | Certificates | CE, UKCA, KC, FCC, IC, RCM, BIS, SIG, RoHS, REACH, WEEE, ETL Listed to UL 294 |
| Interface | RS-485 | 1 ch |
|  | RS-485 Communication Protocol | OSDP V2 compliant |
|  | Wiegand | 1 ch |
|  | LED | 2 ch |
|  | Beep | 1 ch |
|  | Tamper | Supported |
| Electrical | Power | - Voltage: 12 Vdc - Current: Max. 0.3 A |
|  | LED, Beep Input VIH | - Min.: 3V - Max.: 5V |
|  | LED, Beep Input VIL | Max.: 1V |
|  | LED, Beep Pull-up Resistance | 4.7 kΩ (The input ports are pulled up with 4.7 kΩ.) |
|  | Wiegand Output VOH | More than 4.8 V |
|  | Wiegand Output VOL | Less than 0.2 V |
|  | Wiegand Output Pull-up Resistance | Internally pulled up with 1 kΩ |
| Platform | BioStar 2 | Supported |
|  | BioStar 1 | Not Supported |

1) DESFire EV2/EV3 cards are supported by having backward compatibility of DESFire EV1 cards. CSN and smart card functions are compatible with XPass D2.

2) RF read range will vary depending on installation environment.

END OF SECTION

PART 3 - EXECUTION

INSTALLER

- Contractor personnel shall comply with all applicable state and local licensing requirements.

PREPARATION

- Contractor shall avoid locating the reader/controller in a location subject to direct sunlight, dust or soot.

STORAGE

- The device shall be stored in an environment where temperature is in the range of -40 °C ~ 70 °C.

- The device shall be stored in an environment where humidity is in the range of 0 % ~ 95 %, non-condensing.

INSTALLATION

- The device shall be installed in an environment where temperature is in the range of -35 °C ~ 65 °C.

- The device shall be installed in an environment where humidity is in the range of 0 % ~ 95 %, non-condensing.

- All wires shall be run through conduit to prevent failure caused by rodent damage.

Connections between card readers and a door controller shall not exceed 100 meters.

All peripheral devices shall be grounded.

To avoid RF interference, a minimum separation distance must be maintained.

- XPD2-MDB

| - Wall thickness | - Distance |
| --- | --- |
| - 100 mm | - 360 mm |
| - 120 mm | - 360 mm |
| - 150 mm | - 300 mm |

- XPD2-GDB, XPD2-GKDB

| - Wall thickness | - Distance |
| --- | --- |
| - 100 mm | - 400 mm |
| - 120 mm | - 380 mm |
| - 150 mm | - 380 mm |

EXAMINATION

- All connections to the reader/controller shall be tested for proper levels of performance.

END OF SECTION